Let’s separate the CPU operation modes to this document.

🧠 CPU Modes Explained:

Part 1 – Real Mode (The Wild West of Computing)

⚙️ What Even Is a CPU Mode?

Before we get deep, here’s the vibe:

A CPU “mode” is like the operating style of the processor.
It defines what the CPU is allowed to do:

- How much memory it can touch.

- Whether it has access control/security.

- What kind of instructions and registers it can use.

It’s like your CPU switching between “beginner,” “intermediate,” and “pro” modes depending on what it’s trying to run.

🧱 Real Mode: The 16-Bit Legacy Arena

🕰️ This is the original mode of the x86 CPU family. Born with the 8086 processor (1978), and every modern x86 CPU still starts in Real Mode when powered on — even your Core i9 or Ryzen 9.

🔍 Key Traits of Real Mode:

🧪 Addressing Style

Real Mode uses Segment:Offset addressing. It breaks up memory access like this:

🧠 Basically: Segment × 16 (or left-shift 4 bits) + Offset.

That’s how Real Mode squeezes 20-bit memory access out of 16-bit registers.

I know you didn’t get anything, lets revisit this madness about 16-bit real mode.

🧠 Addressing Style: Segment:Offset in Real Mode (The Old-School Memory Map)

You've seen that 16-bit real mode only uses 16-bit registers (like AX, BX).

But wait, how does it access 1MB of memory (which needs 20 bits for its address)?

This is where the Segment:Offset magic comes in.

Think of it like this:

Imagine you're trying to find a specific house in a very old, sprawling city where the streets aren't perfectly numbered, and your GPS (CPU) is a bit limited.

Your GPS (CPU) can only give you 16-bit numbers. That's like your GPS only being able to tell you numbers up to 65,535. That's not enough to cover a whole city with a million houses!

So, they came up with a two-part address system: Segment:Offset.