LADDER LOGIC 2

🤖 Chapter 5-2: Basic Instructions in Ladder Logic

Boolean Logic and the DNA of Control

Now that you know what Ladder Logic looks like, it’s time to understand how it actually decides things. And that takes us straight into Boolean logic — the bedrock of all decision-making in PLCs.

Don’t worry, this isn’t Digital Systems 101 — we’re not breaking out Karnaugh maps or Boolean algebra proofs. We’re just gonna look at the essentials: AND and OR logic.

🔲 Boolean Logic in Plain Language

PLC logic is built on simple True/False decisions — like light switches:

- TRUE (1) → There's voltage or a condition is satisfied

- FALSE (0) → No voltage or the condition isn't satisfied

From this binary setup, we build logic gates. In Ladder Logic, these gates are not separate components — they’re created through how you arrange your rungs and contacts.

🔗 The AND Gate — Series Logic

AND operations are analogous to multiplication.

OR operations are comparable to addition.

🔌 AND gate Ladder Implementation:
Use normally open contacts in series.

This is saying:

“Only if A AND B are both TRUE, then turn ON the output.”

If either A or B is FALSE, current stops flowing — just like if one switch in a series circuit is off.

🔌 OR gate Ladder Implementation:
Use normally open contacts in parallel.

This is saying:

“If A OR B is TRUE, then turn ON the output.”

As long as at least one of them is ON, the rung is complete and current flows to the output.

If A is TRUE (and B is FALSE), the path through A "closes," allowing logical power to flow to the output.

If B is TRUE (and A is FALSE), the path through B "closes," allowing logical power to flow to the output.

If both A and B are TRUE, both paths "close," and logical power still flows to the output.

Only if both A and B are FALSE will both paths remain "open," stopping the logical power flow and keeping the output OFF.